# Setup Procedure for Relay Station (vind-rly) with SSH Tunneling

# Prerequisites

#### Hardware:

Raspberry Pi 5 (or similar) as vind-rly

Wi-Fi adapter (e.g., wlan1) compatible with wfb-ng for the tunnel

Ethernet connection for eth0 to the ground station network (10.5.6.0/24)

Companion computer (e.g., another Raspberry Pi) at 10.5.5.87 with SSH enabled

Ground station device (e.g., a laptop at 10.5.6.50) with an SSH client

Software:

Fresh Ubuntu 24.04 LTS installation on vind-rly

Internet access (initially via wlan0 or eth0) for package installation

Assumptions:

wfb-ng is pre-installed on the companion computer (10.5.5.87) and configured to connect to vind-rly via wlan1

SSH server is running on 10.5.5.87 (user roz)

#### 1. Initial System Setup

Install Ubuntu on vind-rly

Flash Ubuntu 24.04 LTS to an SD card using Raspberry Pi Imager.

Boot the Pi, log in with default credentials (e.g., ubuntu:ubuntu), and change the password:

Set Hostname

sudo hostnamectl set-hostname vind-rly

Create User vind-admin

sudo adduser vind-admin sudo usermod -aG sudo vind-admin

#### 2. Configure DHCP Server on eth0

Install DHCP Server

sudo apt install isc-dhcp-server

Configure DHCP Server

sudo nano /etc/dhcp/dhcpd.conf

Replace contents with your current configuration:

authoritative; default-lease-time 600; 
max-lease-time 7200; 
# Define subnet and assign IP range for DHCP clients 
subnet 10.5.6.0 netmask 255.255.255.0 { 
range 10.5.6.50 10.5.6.100; # Range of IPs to assign dynamically 
option routers 10.5.6.1; 
option domain-name-servers 8.8.8.8, 8.8.4.4; # Google DNS servers 
option broadcast-address 10.5.6.255; 
default-lease-time 600; 
max-lease-time 7200; 
}

Specify the interface:

sudo nano /etc/default/isc-dhcp-server

INTERFACESv4="eth0"

Start DHCP Service

sudo systemctl enable isc-dhcp-server ]

sudo systemctl start isc-dhcp-server 
sudo systemctl status isc-dhcp-server

sudo netstat -uap | grep dhcp # Should show dhcpd listening

Add Delay to DHCP Service

Create an override file to add a 10-second delay:

sudo systemctl edit isc-dhcp-server.service

Paste the following:

[Service]

ExecStartPre=/bin/sleep 10

Save and exit (this creates /etc/systemd/system/isc-dhcp-server.service.d/override.conf with the 	delay).

Reload systemd:

sudo systemctl daemon-reload

Test DHCP

Connect a ground station (e.g., vind-gs) to eth0 via Ethernet or a switch.

On vind-gs:

ip addr show # Should get an IP like 10.5.6.50 ping 10.5.6.100 # Should work

#### 3. Network Configuration

Install Network Tools

sudo apt install net-tools network-manager

Configure netplan

sudo nano /etc/netplan/50-cloud-init.yaml

Paste the following:

network:

version: 2

renderer: networkd

wifis:

wlan0:

optional: true

dhcp4: false

addresses:

- 192.168.1.87/24

# routes:

#   - to: 0.0.0.0/0

#     via: 192.168.1.1

# nameservers:

#   addresses:

#     - 192.168.1.1

access-points:

"Nilan":

auth:

key-management: "psk"

password: "6ab183855b00d5acd59eabd12437edb1caae60016c58bc41391cb1a9b0948124"

ethernets:

eth0:

dhcp4: no

addresses:

- 10.5.6.100/24  # Ethernet IP address in the 10.5.5.x subnet

# - 10.5.6.87/24

optional: true

Apply the configuration:

sudo chmod 600 /etc/netplan/50-cloud-init.yaml

sudo netplan apply

Verify Network

ip addr show eth0 # Should show 10.5.6.100/24 ip addr show wlan0 # Should show 192.168.1.87/24 
ping 192.168.1.1 # Should work if wlan0 connects

Enable IP Forwarding

sudo sysctl -w net.ipv4.ip_forward=1

sudo nano /etc/sysctl.conf

Add the following line:

net.ipv4.ip_forward=1

Apply:

sudo sysctl -p

#### 4. Install and Configure wfb-ng

Install Realtek Patched Driver

Alfa -Wi-Fi Adapter Patch Driver Installation
pip3 install --user --break-system-packages pyroute2 msgpack

sudo apt install python3-msgpack
sudo apt-get install dkms

git clone -b v5.2.20 https://github.com/svpcom/rtl8812au.git

cd rtl8812au/
sudo apt install linux-headers-$(uname -r) – if unsure about kernel version use this otherwise use below command with correct kernel version.

sudo apt install linux-headers-6.8.0-1013-raspi – mention correct kernel version .

sudo ./dkms-install.sh
iw dev wlx00c0cab6db1a info

Rename above Adapter to Wlan1
 
sudo nano /etc/udev/rules.d/70-persistent-net.rules

Add Below Line for wlx00c0cab6db1a

SUBSYSTEM=="net", ACTION=="add", ATTR{address}=="00:c0:ca:b6:db:1a", NAME="wlan1"

Installing Wfb-Ng On Relay Station

git clone https://github.com/svpcom/wfb-ng.git

cd wfb-ng/

sudo ./scripts/install_gs.sh wlan1

Ip route and check for wfb - GS route

vind-admin@vind-rly:~$ ip route

default dev gs-wfb proto static

10.5.5.0/24 dev gs-wfb proto kernel scope link src 10.5.5.77

10.5.6.0/24 dev eth0 proto kernel scope link src 10.5.6.100

192.168.1.0/24 dev wlan0 proto kernel scope link src 192.168.1.87

Relay –station  config file

sudo nano /etc/wifibroadcast.cfg

[path]

conf_dir = '/etc'

bin_dir = '/usr/bin'

tmp_dir = '/tmp'

log_dir = '/var/log'

[common]

wifi_channel = 161

wifi_region = 'BO'

wifi_txpower = 4000

[gs_video]

peer = 'connect://10.5.6.50:5600'

#peer = 'connect://255.255.255.255:5600'

#frame_type = 'data'

#fec_k = 8

#fec_n = 14

#fec_timeout = 0

#fec_delay = 500

bandwidth = 20

short_gi = False

stbc = 1

ldpc = 1

mcs_index = 1

[gs_mavlink]

peer = 'connect://10.5.6.50:14550'

#peer = 'connect://255.255.255.255:14550'

#frame_type = 'data'

#fec_k = 1

#fec_n = 3

#fec_timeout = 0

#fec_delay = 500

bandwidth = 20

short_gi = False

stbc = 1

ldpc = 1

mcs_index = 1

[gs_tunnel]

peer = '10.5.5.87/24'

ifname = 'gs-wfb'

ifaddr = '10.5.5.77/24'

default_route = True

frame_type = 'data'

#fec_k = 2

#fec_n = 4

fec_timeout = 0

fec_delay = 500

bandwidth = 20

short_gi = False

stbc = 1

ldpc = 1

mcs_index = 1

sudo systemctl enable wifibroadcast@gs.service

sudo systemctl enable wifibroadcast.service

sudo systemctl start wifibroadcast.service

sudo systemctl start wifibroadcast@gs.service 
sudo systemctl status wifibroadcast@gs.service

Test Companion Connectivity

ping 10.5.5.87

#### 5. Configure SSH

Install SSH Server

sudo apt install openssh-server

sudo systemctl enable ssh 
sudo systemctl start ssh

#### Set Up Persistent SSH Tunnel

Setting Up Key-Based SSH Authentication to Companion Computer
1. Generate SSH Key
To create an SSH key pair, run:

ssh-keygen -t ed25519 -f ~/.ssh/id_ed25519 -N ""

- -t ed25519 → Uses the secure Ed25519 key type.

- -f ~/.ssh/id_ed25519 → Specifies the key file location.

- -N "" → Creates a key without a passphrase for automation

2. Copy Public Key to Companion Computer

ssh-copy-id -i ~/.ssh/id_ed25519.pub roz@10.5.5.87  
This adds the key to ~/.ssh/authorized_keys on roz.

3. Verify Passwordless SSH Login

ssh -i ~/.ssh/id_ed25519 roz@10.5.5.87
4. Backup SSH Keys
cp ~/.ssh/id_ed25519 ~/.ssh/id_ed25519.pub /path/to/backup/

Install autossh

sudo apt install autossh

Create Log File

sudo touch /var/log/ssh-tunnel.log sudo chown vind-admin:vind-admin /var/log/ssh-tunnel.log 
sudo chmod 644 /var/log/ssh-tunnel.log

Configure Tunnel Service

sudo nano /etc/systemd/system/ssh-tunnel-to-companion.service

Paste the following:

[Unit] Description=SSH Tunnel to Companion Computer 
After=network.target wifibroadcast@gs.service 
[Service] 
ExecStart=/usr/bin/autossh -M 0 -L 0.0.0.0:2222:10.5.5.87:22 roz@10.5.5.87 -N -i /home/vind-admin/.ssh/id_rsa -v 
Restart=always 
User=vind-admin 
Environment=AUTOSSH_LOGFILE=/var/log/ssh-tunnel.log 
Environment=AUTOSSH_DEBUG=1 
StandardOutput=journal 
StandardError=journal 
[Install] 
WantedBy=multi-user.target

Apply the service:

sudo systemctl daemon-reload

sudo systemctl enable ssh-tunnel-to-companion.service 
sudo systemctl start ssh-tunnel-to-companion.service

Verify Tunnel

sudo systemctl status ssh-tunnel-to-companion.service # Active (running) sudo netstat -tuln | grep 2222 # 0.0.0.0:2222 LISTEN

Apply Delay

sudo systemctl edit ssh-tunnel-to-companion.service

Paste:

[Service]

ExecStartPre=/bin/sleep 10

sudo systemctl daemon-reload

sudo reboot

#### 7. Test from Ground Station

From any system on 10.5.6.0/24 (e.g., vind-gs):

ssh roz@10.5.6.100 -p 2222

Windows (PuTTY): Host 10.5.6.100, Port 2222, User roz

macOS/Linux: As above.

#### 8. Optional: Enable Internet via wlan0 (Development)

Edit netplan:
 sudo nano /etc/netplan/50-cloud-init.yaml

Uncomment the routes and nameservers sections:

wifis: wlan0: 
optional: true 
dhcp4: false 
addresses: 
- 192.168.1.87/24 
routes: 
- to: 0.0.0.0/0 
via: 192.168.1.1 
nameservers: 
addresses: 
- 192.168.1.1 
access-points: 
"Nilan": 
auth: 
key-management: "psk" 
password: "February@09" # Replace with your Wi-Fi password

Apply and adjust default route:

sudo ip route del default dev gs-wfb sudo netplan apply 
ping 8.8.8.8 # Confirm internet access

P2P Over Wifi

🔧 Step-by-Step: Wi-Fi Direct (P2P) Auto-Setup on Boot

sudo nano /etc/wpa_supplicant/wpa_supplicant.conf

Add following lines

ctrl_interface=/var/run/wpa_supplicant

update_config=1

device_name=VIND_RLY_P2P

✅ 2. Create the P2P Startup Script

sudo nano /home/vind-admin/rely_p2p.sh

Paste this:

#!/bin/bash

# Start wpa_supplicant on wlan0 with control interface

wpa_supplicant -B -i wlan0 -c /etc/wpa_supplicant/wpa_supplicant.conf -C  /var/run/wpa_supplicant

# Create a temporary P2P group

wpa_cli -i wlan0 p2p_group_add persistent=0

# Wait for interface to be created

sleep 2

# Assign static IP

ifconfig p2p-wlan0-0 10.5.6.101 netmask 255.255.255.0 up

# Start WPS PIN mode (optional for phone pairing)

wpa_cli -i p2p-wlan0-0 wps_pin any 1987

Make the script executable:

sudo chmod +x /home/vind-admin/rely_p2p.sh

✅ 3. Add to crontab for Boot-Time Execution

sudo crontab -e

Add this line:

@reboot /home/vind-admin/rely_p2p.sh

✅ 4. Reboot & Confirm

After reboot: